Battery Cycle Life Test

Summary report for the 18650 cell qualification
programme

1. Purpose

This report summarises the cycle life results collected for the candidate 18650 lithium-ion cell across three
charge rates. The programme's aim was to establish whether the cell retains at least 80 percent of its rated
capacity after 500 full charge-discharge cycles, the threshold this product line requires before a supplier is
qualified for volume orders.

2. Method

Twelve cells were drawn at random from a single production lot and divided into three groups of four, one
group per charge rate: 0.5C, 1.0C and 1.5C. Each cell was cycled between 3.0V and 4.2V at 23C in a
temperature-controlled chamber, with a full capacity check performed every 50 cycles on a calibrated cycler.

Cells that fell outside 2 percent of the group's mean capacity at the first check were flagged and continued on a
separate log, so a single early outlier would not distort the reported group average.

Two cells were withdrawn from the programme before the 500-cycle mark: one for a connector fault unrelated
to the cell itself, and one for a thermal excursion that exceeded the test's safety envelope. Neither withdrawal is
included in the averages below.

3. Results

Table 1 lists the capacity retention measured at the 500-cycle checkpoint for each group, averaged across the
surviving cells in that group.

Charge rate

Cells

Capacity retention

Notes

0.5C